Robust Fuzzy Control of a Model Train

Jörg Kahlert
Ingenieurbüro für Datenverarbeitung und Automatisierungstechnik
Werler Str. 91
D-59063 Hamm
Tel. +49-2381-580419
Fax: +49-2381-580513

Abstract: This paper deals with robust position control of a model train based on linguistic production rules in combination with a very simple sensor unit. Performance indices are exactness and smoothness of the train´s motion especially with regard to the robustness in the case of parameter variations like different load or speed.

I. Introduction

The subway project in Sendai/Japan [1, 2] first proved the efficiency of fuzzy control in the field of vehicle guidance. In comparison with control by man an improvement of riding comfort and energy consumption was obtained. Fundamental for this improvement was a predictive fuzzy control strategy based on the well known mathematical model of the train´s dynamics.

In contrast to that project this paper presents an application of fuzzy control where no mathematical model is required. In particular the robustness of the fuzzy controller in case of parameter variations has to be considered (cf. [3, 4]).

II. Principle and structure

First of all the control system consists of the plant - a model train driven by a DC motor. The nonlinear dynamics of the train results on the one hand from nonlinearities of the motor, on the other hand - and that´s fundamental - from the friction between the train´s wheels and the rail (cf. [5]). This friction effects that the standing train needs a minimum voltage umin> 0 for getting started. The value of umin - just as the maximum speed - depends on the load of the train (see Fig. 1).

The measurement of the train´s position x - or strictly speaking the distance to the desired stopping point - is realized by an optical pulse generator mounted at the train´s axle. The generator´s resolution is 1 Pulse/0.94 cm. The number of generated pulses in this way is proportional to the actual position of the train, while the interval between two pulses can be used for the estimation of the train´s speed. The information is transmitted by an infrared interface.

The fuzzy controller algorithm is implemented on an ordinary personal computer with a sampling time of about 5 ms. Inputs of the controller are position x and (estimated) speed µ § of the train, control output is the motor voltage u generated by a D/A-converter followed by a power amplifier. Fig. 2 shows the structure of the fuzzy controller with the components fuzzification, inference and defuzzification, Fig. 3 an outline of the control system´s components.

Fig. 1: Dependance of umin on train mass m.

Fig. 2: Structure of the fuzzy controller.

Fig. 3: Structure of closed loop system.

III. Fuzzy controller design

For the fuzzification of the controller´s inputs and output in each case five linguistic terms denominated very_small (00), small (0), medium (+), big (++) and very_big (+++) were defined (Fig. 4). We can recognize that the support of the fuzzy sets is getting smaller when the train reaches the stopping point - that means, the resolution of the fuzzy controller increases. This choice produces the nonlinear characteristics of the controller in the main.

Fig. 5 shows the rule matrix of the controller. For example we can find the rules

IF position=small	AND speed=small	THEN output=medium	(line 2, column 2)     or

IF position=small	AND speed=big	THEN output=small	(line 4, column 2).

Also we can see that in the case of very big and very small distance from stopping point the speed of the train does not matter. For a "normal" stopping process only the shaded rules are of interest.

Fig. 4: Fuzzy sets for position, speed and control output.

Fig. 6 shows the resulting input-output characteristic of the fuzzy controller on the condition of max-min-inference and defuzzification by the centre-of-gravity method.

Fig. 5: Rule matrix of the fuzzy controller.

Fig. 6: Input-output characteristic.

IV. Results

The efficiency of the fuzzy control strategy is demonstrated by Fig. 7. It illustrates the stopping process of the train for three different values of the train load m, characterized by position, estimated speed and control output. To obtain the same scaling of axes for all variables, the position x is recorded only for values x < 1m.

First of all the noisy trajectory of the speed is conspicuous. It results from the estimation of the train´s speed by measuring the interval µ § between two pulses of the pulse generator. Because of manufacturing tolerances µ § is not constant even if the train is riding with exactly constant speed. This measurement error however has no visible influence on the movement of the train. The general decrease of speed in the interval of 1.5-2.5 s results from a narrow curve at the beginning of the stopping range. The stopping process itself starts after about 2.5-3 s and - as required - the train slows down very smoothly. The remaining error is in all three cases less than the resolution of the position measurement, 0.94 cm.

Fig. 7: Trajectories for minimum load m = 0 % (a), m = 30 % (b) and maximum load m = 100 % (c).

V. Conclusions

This paper shows that especially in the case of highly nonlinear processes with parameter variations the design of a fuzzy controller leads to a closed loop system with good performance over a wide range of system parameters. The principles used here only in connection with a model system may be transferred to a large field of real systems.

References

[1]	Automatic Train Operation. Techno Japan, Vol. 23, No. 3, March 1990

[2]	Sato, I.: Sendai Municipal Subway Started by Opening of Its North-South Line. Japanese Railway Engineering, No. 104, December 1987

[3]	Kahlert, J.; Frank, H.: Robuste Positionsregelung durch Fuzzy Control am Beispiel eines Eisenbahn-Modells. VDE-Fachtagung "Technische Anwendungen von Fuzzy-Systemen", Dortmund 1992

[4]	Kahlert, J.; Frank, H.: Fuzzy-Logik und Fuzzy-Control. Vieweg-Verlag Wiesbaden 1993

[5]	Palm, R.: Fuzzy Controller for a Sensor Guided Robot Manipulator. Fuzzy Sets and Systems (31) 1989